The Lidar instrument for measuring

The parameters necessary for understanding physics and chemistry 
in the atmosphere

Philippe Keckhut, Slimane Bekki

(LATMOS, France)

Necessary tools :

1- Position the students in front of a wall with 2 lines on the ground, one at 2 meters from the wall at a distance of 4 meters, having a ball and a stopwatch.

2- A laser (ordinary laser pointer) and a normal office lamp, chalk powder, in a dark room.

## PART 1 Objectives

## By analogy with common things, we will understand two important parts in the operation of the lidar, the measurement of the distance and the measurement of the density of what is measured. For ozone, for example, density is the number of molecules contained in the atmosphere at an altitude (distance from the ground) z.

The Lidar, in English Light Detection and Ranging makes it possible to measure many things in the atmosphere of the Earth but also of other planets: ozone, temperature, dust, clouds etc ... A green ray is emitted to the sky Or blue, or red or invisible to the eye) and one looks at the amount of light that is reflected by the atmosphere. In this way several properties are deduced at each altitude, following a principle of measurement which we will describe in the second part.

### PART 2  Understand the 2 important principles concerning the operation of LIDAR

1- Principle of distance calculation: to understand the principle of the altitude measurement with the lidar, hit the balloon when it is at 2 m then 4 meters from the wall. Measure the time taken by the ball to go back and forth t.

The distance from the line to the wall D is thus: v.t = 2. D or v is the speed of the balloon assuming that the balloon is given the same initial velocity in both cases. The distance D is thus PROPORTIONAL to the time taken by the ball to return to its starting point.

2- Principle of density calculation (or other parameter): In the dark room, turn on the laser (attention to the safety instructions) towards the wall or note the image of the beam (green dot for a green laser) . On the path of the beam, project the chalk powder, first very little, then more and more. We position ourselves next to the laser to analyze this experiment. The light reflected by the powder varies as a function of the amount of chalk soldered on the beam, describe how. Does the image on the wall show changes too?

Help: The more bright a signal is, the higher the light intensity.

3- Link with the lidar: what does v correspond to in the equation v x t = 2 x D. How is this equation written for the lidar and altitude z of observation? Interpret the figure below according to what you understood. How will the number of particles (clouds, ozone, dust, etc.) in the atmosphere be measured?